### SINAMICS LAB HMI (ProTool) Interface

### Exercise:	Configuration of the drive unit with Servo control with an HMI (ProTool) Interface

### Using the training-case SINAMICS S120

Task 1:	Online / Offline configuration of a SIMOTION SCOUT / STARTER – project and download to the target system

##### CU320

RAM

##### Scout or Starter

##### ROM

Load into target system

Save to ROM

NOTE:

This HMI lab can be started from the SERVO lab #1 as a starting point.

Open your completed Lab #1 and Save As HMI

Offline configuration of SERVO_02

Double Click [ Configuration ]

Click [ Configure DDS ]

This lab requires the use of Extended Setpoint Channel to address the speed setpoints via the HMI.

Configure DDS sequence:

Select [ Continue ]						Select [ Continue ]

Accept all the remaining configurations as set up from Lab #1 exercise.

Repeat this same configuration [ Extended setpoint channel ] for SERVO_03

Save and compile

Go back Online

Select [  Download ]

Open the [ Control Logic ] configuration for SERVO_02 and select p840[0] BI: ON/OFF as R2094: Bit 0, BO: PROFIBUS PZD receive.

This was selected based on the Function Diagram:

Figure 2-54 2460 - Receive telegram, free interconnection via BICO (p0922 = 999)

The next page shows the connection between the parameter written to by the HMI and the r2094 parameter selected by p840[0] BI: ON/OFF1

This is an excerpt from the Function Diagrams of the S120 Manual:

The HMI application is configured to write a 1 or 0 to p2098[0].0 which in turn will gate a 0 or 1 to r2094.0 which will activate or deactivate the ON/OFF1 bit for the SERVO

The above function diagram can be viewed on page 2-777 of the Parameter List & Functions Sinamics manual:  SINAMICS_S_Listenhandbuch_0605_eng.pdf

Open the >> Setpoint Channel – Speed Setpoint configuration for SERVO_02

Select p1001[0], CO; Fixed speed setpoint

Open the >> Setpoint Channel – Speed Setpoint configuration for SERVO_03

Select p1001[0], CO; Fixed speed setpoint

NOTE:

The p1001[0] parameter selected for SERVO_02 and p1001[0] are unique for PROFIBUS mapping.  The next section details how the HMI will send data to the p1001[0] (REAL) registers using the PROTOOL interface.

ProTool Tag Addressing for communications with Sinamics is accomplished through drive objects.  The Sinamics demo has three drive objects.

1

2

3

Drive Objects:

1 Drive Object 1 = CU320

DBW : Data Block Word 1024

2 Drive Object 2 =  Sinamics Drive 1

DBW : Data Block Word 2048

3 Drive Object 3 = Sinamics Drive 2

DBW : Data Block Word 3072

The in feed module (SLM) does not count as a drive object.

Example Tag configuration for Drive Object 1 : CU320

1 Drive Object 1 = CU320

DBW : Data Block Word 1024

Drive Object 1 = CU320

DBW : Data Block Word 1024

The CU320 Digital inputs are mapped into parameter 722.  The ProTool tag is configured as:

Range:  DB

DB:  722

DBW:  1024

Example Tag configuration for Drive Object 2 : SERVO_02

2 Drive Object 2 =  Sinamics Drive 1

DBW : Data Block Word 2048

Drive Object 2 = SERVO_02

DBW : Data Block Word 2048

SERVO_02 Control Word is parameter 898

Range:  DB

DB:  898

DBW:  2048****

****Above Indicia in example is = 0. DBW will equal 2048 + Indicia # (0). For example--- for Indicia 1 the DBW value would be 2049.

Example Tag configuration for Drive Object 3 : SERVO_03

3 Drive Object 3 = Sinamics Drive 2

DBW : Data Block Word 3072

Drive Object 3 = SERVO_03

DBW : Data Block Word 3072

SERVO_03 Speed set point is mapped to parameter 1001

Range:  DB

DB:  1001

DBW:  3072

Example Tag configuration for Drive Object 3 : SERVO_03

Drive Object 3 = SERVO_03

DBW : Data Block Word 3072

SERVO_03 p840(0) BI: ON/OFF1 is mapped (via a gate) through p2098 onto r2094

Range:  DB

DB:  2098

DBW:  3072